31r6 Second Assignment

UNIVERSITY OF STIRLING

Department of Computer Science and Mathematics

The Dlx Processor

9851205

31R6 Second Assignment

The Dlx Processor

Contents

Title	Page

Why does the DLX pipeline require a RISC instruction set	1

What Addressing Modes can DLX support	4

What extra functional units would allow DLX to support more complex

addressing modes	6

What is the best way to make an extended DLX processor support legacy

CISC instructions	8

Bibliography	10

Why does the dlx pipeline require a RISC instruction set

The DLX Processor has a five-stage pipeline. These stages are the Instruction Fetch (IF), Instruction Decode (ID), Execute (EX), Memory Access (MEM), and Write Back (WB). The width of the pipeline is only 32 bits. This means that the maximum amount of memory that can be loaded into the pipeline each clock cycle is 32 bits. This fixed restriction, means that all instructions are the same size, one of the main restrictions of the RISC architecture. (C.f. Esponda et al).

Source: www.iastate…DLXpipe.html

A fixed instruction format also makes the decoding of instructions easier (Esponda et al). The DLX Processor reserves 6 bits for opcodes, 5 bits for register operations, and the remaining bits for any extra information required by the instruction. Decoding is done in parallel with reading registers, which is possible because these fields are a fixed location in the DLX instruction format (www.cs.iastate…DLXimplem.html).

The Write Back (WB) part of the DLX pipeline provides the DLX pipeline the ability to store registers back into the Execute part of the pipe. This provides the Arithmetic Logic Unit at the EX stage with the latest version of the registers so that Register-Register operations can be carried out quickly. At first thought, it appears that there could be a great problem as the EX and WB stages are separated by the Memory Access stage. However not every instruction uses the MEM stage. In fact Register-to-Register operations don’t use the MEM stage at all. Thus the Write Back can write the registers back to the EX stage to speed the pipe up.

The Register-to-Register operations increase the overall speed of the pipe, as the stages aren’t slowed down by memory fetch/write operation. This importance of the pipeline speed and such restriction is a key part of the RISC architecture. This architecture demands that all instructions must finish in one complete cycle without stalling, and those which cannot must be infrequently used and be the programs compiled in such a way that the stalling can be reduced or even removed. One classic example is the following:

a = b + c ; d = e + f;

If this c code were compiled exactly as it appears, then the code would look like;

LW R1,B

LW R2,C

ADD R4,R1,R2 			// this causes a stall as the value of R2 won’t be ready yet.

SW A,R4				// this causes a stall as the value of R4 won’t be ready yet.

LW R3,E

LW R5,F

ADD R6,R3,R5			// this causes a stall as the value of R5 won’t be ready yet.

SW A,R6				// this causes a stall as the value of R6 won’t be ready yet.

If instead you rearrange the code to do exactly the same thing, but in a different, yet legal order, you will mostly avoid stalling (1 stall instead of 4);

LW R1,B

LW R2,C

LW R3,E

ADD R4,R1,R2	// Here R2 now is available (owing to the 1 stage gap between the EX and WB stages).

LW R5,F

SW A,R4	// as above, R4 is now available due to the 1 stage gap.

ADD R6,R3,R5	// Again, R5 is now available due to the 1 stage gap.

SW D,R6	// this will cause a stall as the value of R6 won’t be ready yet.

The DLX architecture means that it has Load-Store architecture. In fact it has a Three-operand, load-store architecture (www.cs.nmsu…dlx.html). It either operates on registers, or loads and stores, and very little else. There is little scope for Load/Store combined with registers, other than the basic storing and loading of the registers themselves. The DLX solves a problem with stalling after loads with a technique called “Delayed Load”. This basically means that the result of a load is not made available immediately, but held back for a cycle. This means that an instruction following a load cannot use the register just loaded, and this condition can be enforced in the compiler, to prevent this confusion.

The DLX processor also has lots of registers (31 general purpose registers, and an always-0 R0), where R31 is used as the link register for in the JAL (jump and link) instruction (www.cs.nmsu…dlx.html). Esponda et al argue that lots of registers are another key point of RISC architecture. As a RISC processor cannot support complex instruction sets, or large numbers of addressing modes, then to compensate this shortcoming lots of registers should be available. This reliance on registers also makes for the RISC processor to be fast, as registers are on chip, very fast memory (even quicker than cache memory). This compensation means that those complex instructions that RISC cannot directly support can be supported through software, at only a small speed disadvantage. However, it can be argued that if you restrict yourself to producing code for a RISC processor, then you will write code more suited to a reduced instruction set anyway. The extra functionality in complex instruction sets can be thought of as expensive luxuries.

Depending upon which revision of the DLX processor you examine, you can also gain 32 more registers, the 32 32-bit floating point registers, which can be combined in pairs, to give 64-bit double precision floating point registers. These registers then require extra execute units in parallel to the integer ALU for floating point operations. One disadvantage of incorporating the FPALU’s is the inherent speed decrease of floating point calculations. Basic FP addition and subtraction can take up to 3 times the time to compute than integer calculations, FP multiplication can take 6 times more, and division 14 times longer (data taken from Prof. Smith Lecture Notes). This means that the pipe will be stalled by 3, 6, or even 14 stages. This extra addition actually goes against Patterson’s design philosophies:

- Instructions must run at the fastest possible rate (without intermediate interpreting levels like microcode)

- Pipelining is more important than program size

Source: Esponda et al after Patterson [1985]

Taking the second of these two points, that pipelining is more important than program size, it is clear to see that if we incorporate a FP unit to the DLX, we get a pipeline that can stall for a considerable length of time if there are any floating point instructions used. However, the DLX floating point unit is IEEE-754 compatible (Sonza Reorda). The program code for the floating-point operations in IEEE-754 is easy to get and convert to any processor (by simply using the specialist compiler for that particular processor). This will produce a program whose code is large, but will allow for a much leaner, less stalling pipe-line, which will make the instructions run at the fastest possible rate, satisfying the above two Patterson RISC points.

In fact, when you compare the Basic DLX Processor with the four Patterson RISC points (given below), the DLX satisfies all four points, with only the fourth point being open to change:

- The instruction set must be kept simple

The instruction set of the DLX contains only 64 opcodes, a register set of 32, an accessible memory size of 32-bits, giving a fixed instruction size of 32-bits.

- Instructions must be run at the fastest possible rate (without intermediate interpreting levels like microcode)

The basic DLX pipeline includes only an integer ALU, and memory access stages at IF, and MEM. The slowest stage then slows the pipe to that speed, but the memory access is still faster than any floating-point operations, especially when a cache is utilised.

- Pipelining is more important than program size

The pipe is kept simple, with only basic addressing modes, and in the basic model of the DLX processor, only integer operations are supported. Also only basic branching is supported, where complex jumps are utilised by using helping instructions previous to the branch. This increases the size of the program, but increases the pipe throughput.

- Compiler technology is a critical ingredient of RISC designs: optimising compilers must transport so much complexity from the hardware into the compiling phase as possible.

- The compilers of any programming language to DLX machine code must arrange code to take advantage of the delayed loading as described above, and also a RISC phenomenon known as Delayed Branching.

Source: Esponda et al after Patterson [1985]

What Addressing MOdes can DLX support

The DLX architecture only supports two addressing modes:

- Immediate

- Displacement

Source: Sonza Reorda

This is not quite the case however, the DLX processor does natively support Register addressing mode also.

Register addressing mode can be shown with the command Add R4, R3 where Register 4 takes the value of Register 4 + Register 3. This is useful when a value is held in a register.

Immediate addressing mode is where a constant value is used. This constant is actually encoded in the 32-bit instruction itself. The I-type instruction takes the format

| 6-bits | 5-bits | 5-bits | 16-bits |  |
| --- | --- | --- | --- | --- |
| Opcode | Register Source. | Register Destination |  | Constant Value |

Source: Sonza Reorda

An example of the immediate addressing mode can be shown with the command
Add R4, #3 where Register 4 takes the value of Register 4 + the value 3. Immediate addressing is useful for constants.

Displacement addressing mode can be shown with the command Add R4, 100(R1) where Register 4 takes the value of Register 4 + the contents of the memory location pointed to by Register 1 offset by positive 100. This mode is useful for accessing local variables, such as those known only by the current method in Java or C++ for example.

M. Sonza Reorda also asserts that the DLX can indirectly support two further addressing modes with useful tricks.

Register deferred addressing can be obtained from the displacement addressing mode by placing 0 in the displacement field. This effectively gives the possibility of coding Add R4, (R1) by in fact coding it as Add R4, 0(R1). Register deferred addressing is useful for accessing using a pointer or a computed address.

Also, Absolute/Direct addressing can be obtained by using R0 as a base register for displacement addressing mode. Remember that R0 is always at value 0. This then gives the ability to code Add R4, (n) (where n is an integer) by in fact coding it as Add R4, n(R0). It must be noted though, that this addressing mode does not give access to the full 32-bit memory. The greatest amount of memory that can be accessed by using R0, uses the I-type instruction above. The constant value here is actually computed as a displacement, and is only 16-bit. Further to this is the consideration is that the value could be considered as a 2’s complement displacement by the compiler. If the compiler does assume the given value to be signed, then the memory that can be accessed in this way will be 0 to 215-1 and 232-215 to 232-1. If the compiler assumes the value to be unsigned, then the memory range will be 0 to 216-1. Either way, a hole of 216 memory locations cannot be accessed with this addressing mode.

Source for Addressing Mode Information: www.cs.iastate…addressMode.html

The remaining addressing modes are listed in the table below. These modes cannot be directly or indirectly (without need for more than a single instruction) supported.

| ##### Addressing modes | Example Instruction | Meaning (using Register Transfer Language) | When Used |
| --- | --- | --- | --- |
| Indexed | Add R3, (R1 + R2) | R3<-R3+M[R1+R2] | Useful in array addressing: R1 – base of array R2 – index amount |
| Memory deferred | Add R1, @(R3) | R1<-R1+M[M[R3]] | If R3 is the address of a pointer p, then mode yields *p |
| Auto-increment | Add R1, (R2)+ | R1<-R1+M[R2] R2<-R2+d | Useful for stepping through arrays in a loop. R2 – start of array d – size of an element |
| Auto-decrement | Add R1, -(R2) | R2<-R2-d R1<-R1+M[R2] | Same as auto-increment. Both can be used to implement a stack as push and pop |
| Scaled | Add R1, 100(R2)[R3] | R1<-R1+M[100+R2+R3*d] | Used to index arrays. May be applied to any base-addressing mode in some machines. |

Source: www.cs.iastate…addressMode.html

It can be worked out however, that one can easily code these addressing modes using the given addressing modes and instructions. For example, if one wanted to code the longest addressing mode, the scaled mode, then the code would look something like this:

LW R4, d				Load the displacement into a temporary Register R4

(using immediate addressing mode)

MULT R4,R3,R4		Multiply R4 (d) by R3, and store the result in R4

(using register addressing mode)

ADD R4,R2				Add R2 to R4 (using register addressing mode)

ADD R1,100(R4)		Add the contents of the memory location R4+100 to R1

(using displacement addressing mode)

Source for Instruction Set: www.cs.iastate…DLXinstrSet.html

This 4 line code achieves the same thing as Add R1, 100(R2)[R3] and was worked out using the basic structure of the Register Transfer Language. However, what the code hides is the inherent stalls that will occur at every stage of the code. So although the code appears to only run at 1/4 of the speed of the scaled addressing mode, but there will be a 1 stage stall at every point in the code, and thus the overall speed of the code is in fact 1/8 of the speed. However the software approach will be better used if this speed is still faster than hardware coding the addressing mode.

What EXTRA functional units would allow DLX to support more complex addressing modes

First, we must assume that we will directly support all forms of addressing modes. This means that we would have to support an extra 7 addressing modes (Register Deferred, Absolute/Direct, Indexed, Memory Deferred, Auto-increment, Auto-decrement, and Scaled). This means that we would have to increase the instruction size by 3-bits. The most complex of these being Scalar addressing mode, for example Add R1, 100(R2)[R3]

Given that we now have an opcode size of 9 bits, we now have to cater for 3 registers; each referred to by 5-bit values, giving us a total of 24 bits in the opcode and operands. Finally we have to consider the displacement. If we try and restrict ourselves into the 32-bit instruction window it only leaves us a displacement of 8 bits left. But since all the previous 6-bit codes are already used (i.e. the first 6 bits give us a specific instruction, and the following 3 bits produce an operand), then we have to increase the instruction size by 3-bits. But a 35-bit instruction doesn’t make much sense in computer terms, as we work in multiples of 8. So a 40-bit instruction seems logical. However, the DLX processor requires that addresses be aligned to even values, a 40-bit instruction set actually gives us a 5-byte instruction set. Therefore we have to increase the window to 48-bit.

Given that the new instruction size is 48-bits, we can now cater for a 24-bit displacement, enough to cater for most scalar addressing situations.

Now consider implementing original, native DLX instructions, such as Add R4,R3. This gives us an instruction size of 9-bits for the opcode, and 2*5 bits for the operands. This gives us a total size of 29 bits. The next nearest even multiple of 8 bits is 32 bits. This means that this instruction can be coded in 32 bits, wasting the loading of the last 16 bits.

Given that some instructions can be coded in 32 bits, and some in 48 bits, then we can split the instruction set, to allow variable sized instructions. However variable sized instructions is a trait of CISC processors, not RISC. Variable instruction sizes mean that we have to load more information sometimes than others to execute an instruction. This leads to slow down at the beginning of the pipe, and the pipe will have been stalled while the data was fetched. A solution to this then is to assume that all instructions are 48-bits wide, and the IF stage should obtain 48-bits into the Instruction Register. When the instruction is decoded it turns out to be a 32-bit instruction, then the PC can be reduced by 2 bytes.

This will however cause further stalls if this check is carried out at the ID stage. Thus the check for the 32/48-bit should be carried out at the IF stage. This requires logic to check this requirement, and a decrementer to decrease the PC by 2 bytes if the instruction is a 32-bit instruction. This adds a further time element to the IF stage, which could slow the whole pipe down. Consider though, how often the processor actually will process 48 bit instructions, and one realises that it would be very infrequent, say around 1 in 10 instructions. This then means that 9 out 10 instructions are wasting time getting 2 bytes extra of memory, then reducing the PC to combat this. This time wastage definitely goes against one of Patterson’s RISC points, that instructions must run at the fastest possible rate.

However, assuming that supporting all the addressing modes was still desirable in the DLX, then one would have to incorporate more decoding hardware in the ID phase, to cope with the extra conditions.

To deal with register deferred, indexed, auto-increment, and auto-decrement addressing one would require an extra ALU between the MEM stage and the WB stage. This would cause the register value to be given to the MEM stage, the relevant memory retrieved, and then the ALU operation could then be applied to the original given register.

To deal with the Scaled addressing mode, one would require an extra ALU in the EX stage (if you choose to use a value for d other than 1) or the ALU to be expanded to deal with 3 terms instead of two (if you choose to set the value of d to be 1), as well as an extra ALU between the MEM stage and the WB stage.

This of course only assumes that you want to support all addressing modes on the source of information (i.e. the right hand side of the operand). If you wished to support instructions such as Add 100(R2)[R3],R1 then you would require even more bits in the instruction set width.

What is the best way to make an extended DLX processor support legacy CISC instructions

There are three main ways in which an executable program written for a CISC architecture can be run on the more modern RISC-based processors:

- Software written in a high-level language such as C should of course be re-compiled to produce a new executable for the target architecture.

- A largely automatic translation of assembly language from CISC to RISC assembly language so that it becomes a full ‘native’ program for the target architecture.

- The software can be left in it’s original form (as object code), and run under an emulator program which simulates the instruction set of the original processor family.

Source: Introduction to CISC-RISC porting

If one were to take options 1 or 2, then the resultant code would be close to perfect object code for the RISC processor. However, the original source code would have been written with CISC architecture in mind. Complex instructions are easily coded with a complex instruction set, yet it may be faster to achieve the overall result in RISC using a different approach.

If one were to take the third option, and run the software through an emulator, then you get a wholly slower process. The basic Fetch-Decode-Execute cycle now becomes Fetch-Decode-Execute(repeat as necessary to convert CISC instruction into RISC instruction) then Fetch-Decode-Execute the final converted RISC code. In fact there is a modern example of this. The Apple Macs used to base themselves on the Motorola 68k family of CISC processors. However, the PowerPC RISC processor has been released and the latest Macs are based only on the PowerPC. There is still a huge amount of software that was written for the 68k family, and so that this software doesn’t become obsolete, MicroAPL have released Mimic/68k that emulates the 68000 and 68020 on a PowerPC or MIPS RISC processor.

If you were to take the more traditional approach, and include at hardware level the ability to support legacy CISC instructions, then you would have the situation described in the previous section. This was where you would had to include an extra ALU in the Execute stage of the pipe, and an extra ALU in between the Memory Access stage and the Write Back stage, to support the computation of the effective address, and then the computation in the instruction itself. If one were to take a microcode aspect of solving complex instructions, then you would have to include further bits in the instruction set, to directly manipulate the microprocessor inside the DLX. These would direct the microprocessor to carry out the pre Execution stage address manipulations, and then an ALU post Memory Access stage to then carry out the actual computation of the instruction.

It is possible to program the pipeline of a CISC processor to use the dead time between the load and store of one instruction argument in memory. The microengine works in this case following a load/store model, and it dynamically reschedules the operations needed by the macrocode. This dynamical rescheduling is too expensive compared to the software scheduling used in RISC processors. Software rescheduling must be done only once and then it runs without complex hardware. Dynamical scheduling needs increasing amounts of logic. (Esponda et al).

However, if you were to take the microcode option, then one detracts from one of the Patterson RISC points, that Instructions should be run as fast as possible (without use of such mechanisms as microcode).

It is a fact that RISC and CISC architectures are becoming more and more alike. Many of today’s RISC chips support just as many instructions as yesterday’s CISC chips. The PowerPC 601, for example, supports more instructions than the Pentium. The difference between RISC and CISC chips is getting smaller and smaller (Gerritsen).

Bibliography

[1]	The RISC Concept – A Survey of Implementations. 	1991

Authors: Margarita Esponda and Ra’ul Rojas

http://www.inf.fu-berlin.de/lehre/WS94/RA/RISC-9.html

[2]	The Basic Pipeline for DLX

http://www.dpi.ufv.br/disciplinas/shared/prabhu/Tutorial/PIPELINE/DLXpipe.html

[3]	An Implementation of DLX

http://www.cs.iastate.edu/~prabhu/Tutorial/PIPELINE/DLXimplem.html

[4]	The DLX Architecture

http://www.cs.nmsu.edu/~pfeiffer/classes/573/notes/dlx.html

[5]	31R6 Lecture Notes	2001

Author: Professor Leslie Smith

http://www.cs.stir.ac.uk/courses/31R6/

[6]	The DLX Architecture

Author: M. Sonza Reorda

http://www.cad.polito.it/~sonza/msuic/lectures/dlx.PDF

[7]	Addressing Modes

http://www.cs.iastate.edu/~prabhu/Tutorial/PIPELINE/addressMode.html

[8]	DLX Instruction Set

http://www.cs.iastate.edu/~prabhu/Tutorial/PIPELINE/DLXinstrSet.html

[9]	Introduction to CISC-RISC porting

Author: MicroAPL

http://www.microapl.co.uk/Porting/intro.html

[10]	CISC vs RISC – Which one is better?

Author: Armin Gerritsen

http://cpusite.examedia.nl/docs/cisc_vs_risc.html

Delayed Branching is a technique used in RISC processors to reduce the stalling in pipelines when a branch is reached. Basically, the branch address is calculated, and then the PC is set to that address. The pipe then continues from that PC. However those commands that are still in the pipe after the branch instruction and before the new PC instruction are executed in their entirety. In the DLX processor, there can be up to 4 instructions in the pipe after the branch instruction, all of which are executed.